| Digitalization Industrialization Standardization Collaboration DISC DEXPI Equinor – Aker BP – Aker Solutions - Aibel |  |  |  |
| --- | --- | --- | --- |
| Document title: DISC DEXPI Final Report | Rev: 1.0 | Page: 1 of 23 |

Table of Contents

1	Project Summary	4

2	Gaps	4

2.1	DISC DEXPI 2.0 Validator	4

2.2	Project symbol discrepancies	4

2.3	Heat Tracing definition	4

2.4	Type Codes (Enumeration Literals)	5

2.5	MeasuringLineFunction rule enforcement	5

2.6	Accepted Class/Attribute rule enforcement	6

3	Project Deliverables	7

3.1	Validation tools for DISC DEXPI 2.0	7

3.2	Update of machine-readable DISC DEXPI Profile	7

3.2.1	Updated Symbol library profile for DISC	7

3.2.2	Updated HTML Human Readable Profile	7

3.3	Additional rule definitions within DISC Profile meta model	8

3.4	Blueprint File	8

4	Recommendations for further work	9

4.1	Reference Data:	9

4.2	Technical Recommendations	9

4.3	Promote Use of DEXPI:	9

4.4	Use Cases / Business Value:	9

Appendix A: Symbol updates	10

Terms and Abbreviations

| Term | Explanation |
| --- | --- |
| DISC | Digitalization Industrialization Standardization Collaboration; initiative between Equinor, AkerBP, Aibel & AkerSolutions. |
| PCA | POSC Caesar Association |
| P&ID | Piping and Instrumentation Diagram |
| DEXPI | DEXPI: Data Exchange in the Process Industry |
| NCS | Norwegian Continental Shelf |
| NORSOK | Norsk Sokkels Konkurranseposisjon |
| Draw.io | Free and open-source diagramming tool |
| PnB | Plants & Bytes: software and consulting company involved in DEXPI group. |
| Github | Platform for software development, collaboration, and security. |

# Project Summary

The DEXPI initiative, launched in 2011, aims to establish a common data exchange standard for the process industry. The previous DISC DEXPI projects defined the first version of the DISC requirements and then extended them to support the DEXPI 2.0 XML schema within the 2025 DISC DEXPI profile.

When this 2025 profile was put into use on the Yggdrasil engineering project, it became clear that a validation tool was needed to ensure DISC DEXPI 2.0 file transfers could be verified both at the point of export from the engineering CAD tool and as part of handover quality assurance prior to import into the receiving system.

Several additional gaps were also identified and addressed, including discrepancies in symbol definitions requiring new symbols, new variants, and updates to existing symbols; inconsistencies in heat tracing rule definitions; missing and inconsistently structured type codes; and the need to enforce the MeasuringLineFunction connectivity rule within the profile.

# Gaps

The following sections describe the gaps identified within the scope of this project, together with the resolutions applied.

## DISC DEXPI 2.0 Validator

The absence of a validation tool for the DEXPI 2.0 serialization format was identified as a critical gap following delivery of the 2025 DISC DEXPI profile. A validation tool for the earlier Proteus-serialized format had been provided by DEXPI.org, but no equivalent existed for the new format. It was therefore necessary to develop a new tool capable of both validating and graphically checking DISC DEXPI 2.0 files, ensuring quality control at the point of export and during handover.

Resolution:

New Validation tool for DISC DEXPI 2.0 profile developed and supplied to the companies involved in the project.

## Project symbol discrepancies

Several symbol discrepancies were identified between the 2025 DISC DEXPI profile and the current engineering project. These included missing symbols, missing symbol variants, off-grid symbol sizes/origo and incorrect connection point positions.

Resolution:

Appendix A provides a list of new and adjusted symbols identified by project as gaps from the 2025 DISC DEXPI profile symbol legend that are updated within the DISC DEXPI profile (ver. 0.6).

## Heat Tracing definition

Inconsistent heat tracing rule definitions were identified as a gap. The previous rule relied on inconsistent usage of heat tracing attributes across DEXPI class types.

Resolution:

The profile is now updated to use the standard HeatTracingType attribute for all relevant class definitions to indicate whether the item requires heat tracing.

## Type Codes (Enumeration Literals)

The 2025 profile was missing type codes and had an inconsistent use of the metadata structure for type codes, both of which required resolution.

Resolution:

- In-line flow instrument type codes added:

  - A : Water In Oil Analyser

  - M : Magnetic Flowmeter

- Meta data structure for type codes has been adjusted. This impacts the Condition definition for the symbol variants.

| 2025 Meta data structure | 2026 Meta data structure |
| --- | --- |
| <Data property="Condition"> <String> ValvePosition = 'NO' </String> </Data> | <Components property="Condition"> <Object type="Profile/PropertyValueCondition"> <Data property="Property"> <String> DiscProfile.InformationModel.OperatedValveExtension.ValvePosition </String> </Data> <Data property="Value"> <String> DiscProfile.InformationModel.ValvePosition.NormallyOpen </String> </Data> </Object> </Components> |

## MeasuringLineFunction rule enforcement

A method was needed to enforce the following rule for MeasuringLineFunction usage in order to ensure correct element definitions within the P&ID.

Rule: MeasuringLineFunction must have a ‘Source’ of ProcessSignalGeneratingFunction and also ProcessSignalGeneratingFunction must be the ‘Source’ of only MeasuringLineFunction.

Resolution:

This rule has been applied as part of the Profile meta model and is validated in the validation tool.

## Accepted Class/Attribute rule enforcement

A method was needed to enforce the use of only approved class and attribute definitions to ensure that any receiving system for the DISC DEXPI 2.0 file can correctly identify and manage mapping for incoming class and attribute elements.

Resolution:

This rule has been applied as part of the Profile meta model within the ‘UsageConstraints’ section.

# Project Deliverables

## Validation tools for DISC DEXPI 2.0

The main project deliverable is the validation tools developed to validate the new DEXPI 2.0 xml schema output format together with the DISC DEXPI profile 0.6 version.

To support validation two tools are in the deliverables:

- pnb Toolbox Command Line 2.0.0 beta 1 for DEXPI 2.0 - DISC (profile version 0.6 from June 2026)

  - this tool is a pure command line tool that can be scripted to run against multiple files and will output errors and warnings in an excel format per input file.

- pnb Toolbox GUI 2.0.0 beta 1 for DEXPI 2.0 - DISC (profile version 0.6 from June 2026)

  - this tool provides validation for a single file along with an intelligent graphical presentation of the file. Graphical elements are clickable with links to the conceptual and diagram model tree views.

## Update of machine-readable DISC DEXPI Profile

This new DISC DEXPI Profile (ver 0.6) is updated with symbol updates listed in Appendix A: Symbol updates along with new type codes and heat tracing attribute references.

### Updated Symbol library profile for DISC

Updated DISC DEXPI symbol library delivered in DEXPI 2.0 format as part of the DISC DEXPI profile (ver 0.6).

Ref: Appendix A: Symbol updates to view changes from symbol definitions delivered as part of 2025 project delivery.

Each symbol incorporates details for:

  - Reference ID

  - Symbol standard sizing

  - Symbol origin point

  - Symbol connection points and types

  - Symbol label placement and attribute definitions

  - Dynamic symbols – symbol variants based on attribute values e.g. normally open/closed variants.

The meta model for the definition of these requirements is provided in the project Profile.xml file and must be used to create DISC DEXPI profile compliant symbols. Compliance is required for the explicit interconnectivity required for verification.

### Updated HTML Human Readable Profile

Delivery of updated HTML version of the profile packaged together with the DEXPI standard in HTML format. This allows users to easily navigate between the DEXPI standard and the DISC DEXPI Profile and to clearly identify where the standard has been extended. The HTML version also provides a convenient way to view symbol details, including graphic variants, nodes, label placements, and associated attributes.

## Additional rule definitions within DISC Profile meta model

The Profile meta model was extended to include additional rule definitions. The meta model now provides rules to ensure correct usage of MeasuringLineFunction, allowed Classes and allowed Attributes. These rules are validated using the validation tools.

## Blueprint File

Blueprint files have been updated to use the new symbol reference IDs, updated symbol definitions, and revised node connection points. They are included as part of the project delivery package together with an overview document describing the changes between versions. Blueprint file versions support testing of Persistent ID requirements by class type.

# Recommendations for further work

## Reference Data:

  - Engage with PCA to take ownership of the profile.

  - Work with PCA to complete DISC DEXPI Semantic references.

  - Work with PCA initiative to take ownership of NCS Symbol Legend## (format to be aligned with profile verification requirements)

  - Work with company drafting standards to enhance the foundation for mapping towards DEXPI

## Technical Recommendations

  - ##Update DISC DEXPI profile to use direct pointers to PCA symbol references

  - Collaborate with Management Of Change (Persistent ID) projects

  - Work with DEXPI group to develop NCS instrumentation and actuating electrical models

  - Work with the DEXPI group to ensure profile meta model definitions are implemented as part of the standard DEXPI definition.

## Promote Use of DEXPI:

  - Support inclusion of DEXPI Deliverable as part of new project contracts.

  - Support inclusion of DEXPI deliverables as part of next DISC ‘Show and Tell’.

  - Secure participation of engineering tool vendors.

## Use Cases / Business Value:

  - Develop use cases to demonstrate machine-based validation.

# Appendix A: Symbol updates

The following table lists the symbol updates as part of the DISC DEXPI profile (ver 0.6)

| Update Category | Description |
| --- | --- |
| Size | symbol size has been updated |
| Origin/Bounds | origin has been moved |
| Variant | new variant added |
| NodePositionType | node type was changed |
| Primitives | symbol graphic was changed |
| NodePosition Count | number of connection points was changed |
| Renamed Symbol | symbol reference ID was changed |
| New Symbol | symbol was added |

** Note: those symbols marked with ‘Incorrect Origo’ require adjustment to ensure centered placement of origo. This ‘todo’ adjustment will be reflected in the next version of the profile.

| Symbol 2025 | Symbol 2026 | Variant | Category | Change Detail |
| --- | --- | --- | --- | --- |
| ND0004 | ND0004 | 0 | Size | Size changed: 18.0×12.0 → 18.0×11.0 |
| ND0004 | ND0004 | 0 | Origin/Bounds | Origin changed: MaxX=9.0→9.0, MinX=-9.0→-9.0, MaxY=9.9→2.1, MinY=-2.1→-8.9 |
| ND0004 | ND0004 | 1 | Size | Size changed: 18.0×12.0 → 18.0×11.0 |
| ND0004 | ND0004 | 1 | Origin/Bounds | Origin changed: MaxX=9.0→9.0, MinX=-9.0→-9.0, MaxY=9.9→2.1, MinY=-2.1→-8.9 |
| ND0004 | ND0004 | 2 | Variant | New variant in 2026 |
| ND0004 | ND0004 | 0 | Incorrect Origo | Incorrect Origo: MaxX=9.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0004 | ND0004 | 1 | Incorrect Origo | Incorrect Origo: MaxX=9.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0004 | ND0004 | 2 | Incorrect Origo | Incorrect Origo: MaxX=9.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0005 | ND0005 | 0 | Size | Size changed: 26.0×12.0 → 26.0×11.0 |
| ND0005 | ND0005 | 0 | Origin/Bounds | Origin changed: MaxX=9.0→17.0, MinX=-17.0→-9.0, MaxY=9.9→2.1, MinY=-2.1→-8.9 |
| ND0005 | ND0005 | 1 | Variant | New variant in 2026 |
| ND0005 | ND0005 | 2 | Variant | New variant in 2026 |
| ND0005 | ND0005 | 0 | Incorrect Origo | Incorrect Origo: MaxX=17.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0005 | ND0005 | 1 | Incorrect Origo | Incorrect Origo: MaxX=17.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0005 | ND0005 | 2 | Incorrect Origo | Incorrect Origo: MaxX=17.0, MaxY=2.1, MinX=-9.0, MinY=-8.9 |
| ND0008A | ND0008A | 0 | NodePosition Type | NodePosition[0].Type: Profile/NodePositionType.Instrumentation → Profile/NodePositionType.Piping |
| ND0026 | ND0026 | 0 | Size | Size changed: 2.0×2.0 → 1.28×1.28 |
| ND0026 | ND0026 | 0 | Origin/Bounds | Origin changed: MaxX=2.0→1.28, MinX=0.0→0.0, MaxY=1.0→0.64, MinY=-1.0→-0.64 |
| ND0026 | ND0026 | 0 | Primitives | Primitive[0].FillStyle (Polygon): ('DataReference', 'Core/Diagram.FillStyle.Transparent') → ('DataReference', 'Core/Diagram.FillStyle.Solid') |
| ND0026 | ND0026 | 0 | NodePosition Count | NodePositions count: 2 → 1 |
| ND0026 | ND0026 | 1 | Size | Size changed: 2.0×2.0 → 1.28×1.28 |
| ND0026 | ND0026 | 1 | Origin/Bounds | Origin changed: MaxX=2.0→1.28, MinX=0.0→0.0, MaxY=1.0→0.64, MinY=-1.0→-0.64 |
| ND0026 | ND0026 | 1 | Primitives | Primitive[0].FillStyle (Polygon): ('DataReference', 'Core/Diagram.FillStyle.Solid') → ('DataReference', 'Core/Diagram.FillStyle.Transparent') |
| ND0026 | ND0026 | 1 | NodePosition Count | NodePositions count: 2 → 1 |
| PA004A | ND0052 | 0 | Origin/Bounds | Origin changed: MaxX=1.0→1.5, MinX=-2.0→-1.5, MaxY=0.0→0.0, MinY=-7.0→-7.0 |
| PA005A | ND0053 | 0 | Origin/Bounds | Origin changed: MaxX=7.0→7.5, MinX=-8.0→-7.5, MaxY=11.4→11.4, MinY=-10.6→-10.6 |
| PA005A | ND0053 | 0 | Incorrect Origo | Incorrect Origo: MaxX=7.5, MaxY=11.4, MinX=-7.5, MinY=-10.6 |
| PD001A | ND0057 | 0 | Origin/Bounds | Origin changed: MaxX=66.0→66.5, MinX=-10.0→-9.5, MaxY=71.8→71.8, MinY=-56.2→-56.2 |
| PD001A | ND0057 | 0 | Incorrect Origo | Incorrect Origo: MaxX=66.5, MaxY=71.8, MinX=-9.5, MinY=-56.2 |
| PD020A | ND0076 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=4.1, MinX=-4.0, MinY=-3.9 |
| PE017A | ND0094 | 0 | Incorrect Origo | Incorrect Origo: MaxX=0.0, MaxY=11.2, MinX=-64.0, MinY=-10.8 |
| PF001A | ND0121 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF002A | ND0122 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF003A | ND0123 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF004A | ND0124 | 0 | Origin/Bounds | Origin changed: MaxX=4.0→4.0, MinX=-4.0→-4.0, MaxY=3.9→4.0, MinY=-4.1→-4.0 |
| PF004A | ND0124 | 0 | NodePosition Count | NodePositions count: 2 → 5 |
| PF005A | ND0125 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF006A | ND0126 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF007A | ND0127 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PF008A | ND0128 | 0 | Incorrect Origo | Incorrect Origo: MaxX=4.0, MaxY=3.9, MinX=-4.0, MinY=-4.1 |
| PS001A | ND0153 | 0 | Size | Size changed: 4.0×14.0 → 3.0×11.0 |
| PS001A | ND0153 | 0 | Origin/Bounds | Origin changed: MaxX=2.0→3.0, MinX=-2.0→0.0, MaxY=2.0→2.0, MinY=-12.0→-9.0 |
| PS001A | ND0153 | 1 | Size | Size changed: 4.0×14.0 → 3.0×11.0 |
| PS001A | ND0153 | 1 | Origin/Bounds | Origin changed: MaxX=2.0→3.0, MinX=-2.0→0.0, MaxY=2.0→2.0, MinY=-12.0→-9.0 |
| PS003A | ND0154 | 0 | Size | Size changed: 4.0×11.0 → 3.0×8.5 |
| PS003A | ND0154 | 0 | Origin/Bounds | Origin changed: MaxX=2.0→3.0, MinX=-2.0→0.0, MaxY=2.0→2.0, MinY=-9.0→-6.5 |
| PS004A | ND0155 | 0 | Size | Size changed: 4.0×11.0 → 3.0×8.5 |
| PS004A | ND0155 | 0 | Origin/Bounds | Origin changed: MaxX=2.0→3.0, MinX=-2.0→0.0, MaxY=2.0→2.0, MinY=-9.0→-6.5 |
| PS006A | ND0157 | 0 | NodePosition Count | NodePositions count: 0 → 1 |
| PT002A | ND0168 | 0 | Incorrect Origo | Incorrect Origo: MaxX=16.0, MaxY=48.1, MinX=-16.0, MinY=-47.9 |
| PT005A | ND0171 | 0 | Incorrect Origo | Incorrect Origo: MaxX=16.0, MaxY=47.9, MinX=-16.0, MinY=-0.1 |
| PV001A | ND0178 | 0 | NodePosition Count | NodePositions count: 0 → 2 |
| PV021A | ND0194 | 0 | NodePosition Count | NodePositions count: 0 → 2 |
| PV022A | ND0195 | 0 | NodePosition Count | NodePositions count: 0 → 2 |
| PZ010A | ND0205 | 0 | Origin/Bounds | Origin changed: MaxX=32.0→32.5, MinX=0.0→0.5, MaxY=0.0→0.0, MinY=0.0→0.0 |
| PZ011B | ND0206B | 0 | Incorrect Origo | Incorrect Origo: MaxX=6.0, MaxY=0.1, MinX=-6.0, MinY=-7.9 |
| PZ013A | ND0209 | 0 | NodePosition Count | NodePositions count: 1 → 2 |
| ND0048 | LZ009A | * | Renamed Symbol |  |
| ND0049 | PA001A | * | Renamed Symbol |  |
| ND0050 | PA002A | * | Renamed Symbol |  |
| ND0051 | PA003A | * | Renamed Symbol |  |
| ND0052 | PA004A | * | Renamed Symbol |  |
| ND0053 | PA005A | * | Renamed Symbol |  |
| ND0054 | PA006A | * | Renamed Symbol |  |
| ND0055 | PA007A | * | Renamed Symbol |  |
| ND0056 | PA008A | * | Renamed Symbol |  |
| ND0057 | PD001A | * | Renamed Symbol |  |
| ND0058 | PD002A | * | Renamed Symbol |  |
| ND0059 | PD003A | * | Renamed Symbol |  |
| ND0060 | PD004A | * | Renamed Symbol |  |
| ND0061 | PD005A | * | Renamed Symbol |  |
| ND0062 | PD006A | * | Renamed Symbol |  |
| ND0063 | PD007A | * | Renamed Symbol |  |
| ND0064 | PD008A | * | Renamed Symbol |  |
| ND0065 | PD009A | * | Renamed Symbol |  |
| ND0066 | PD010A | * | Renamed Symbol |  |
| ND0067 | PD011A | * | Renamed Symbol |  |
| ND0068 | PD012A | * | Renamed Symbol |  |
| ND0069 | PD013A | * | Renamed Symbol |  |
| ND0070 | PD014A | * | Renamed Symbol |  |
| ND0071 | PD015A | * | Renamed Symbol |  |
| ND0072 | PD016A | * | Renamed Symbol |  |
| ND0073 | PD017A | * | Renamed Symbol |  |
| ND0074 | PD018A | * | Renamed Symbol |  |
| ND0075 | PD019A | * | Renamed Symbol |  |
| ND0076 | PD020A | * | Renamed Symbol |  |
| ND0077 | PE001A | * | Renamed Symbol |  |
| ND0078 | PE002A | * | Renamed Symbol |  |
| ND0079 | PE003A | * | Renamed Symbol |  |
| ND0080 | PE004A | * | Renamed Symbol |  |
| ND0081 | PE005A | * | Renamed Symbol |  |
| ND0082A | PE006A | * | Renamed Symbol |  |
| ND0082B | PE006B | * | Renamed Symbol |  |
| ND0084 | PE007A | * | Renamed Symbol |  |
| ND0085 | PE008A | * | Renamed Symbol |  |
| ND0086 | PE009A | * | Renamed Symbol |  |
| ND0087 | PE010A | * | Renamed Symbol |  |
| ND0088 | PE011A | * | Renamed Symbol |  |
| ND0089 | PE012A | * | Renamed Symbol |  |
| ND0090 | PE013A | * | Renamed Symbol |  |
| ND0091 | PE014A | * | Renamed Symbol |  |
| ND0092 | PE015A | * | Renamed Symbol |  |
| ND0093 | PE016A | * | Renamed Symbol |  |
| ND0094 | PE017A | * | Renamed Symbol |  |
| ND0095 | PE018A | * | Renamed Symbol |  |
| ND0096 | PE019A | * | Renamed Symbol |  |
| ND0097 | PE020A | * | Renamed Symbol |  |
| ND0098 | PE021A | * | Renamed Symbol |  |
| ND0099 | PE022A | * | Renamed Symbol |  |
| ND0100 | PE023A | * | Renamed Symbol |  |
| ND0101 | PE024A | * | Renamed Symbol |  |
| ND0102 | PE025A | * | Renamed Symbol |  |
| ND0103 | PE026A | * | Renamed Symbol |  |
| ND0104 | PE027A | * | Renamed Symbol |  |
| ND0105 | PE028A | * | Renamed Symbol |  |
| ND0106 | PE029A | * | Renamed Symbol |  |
| ND0107 | PE030A | * | Renamed Symbol |  |
| ND0108 | PE031A | * | Renamed Symbol |  |
| ND0109 | PE032A | * | Renamed Symbol |  |
| ND0110 | PE033A | * | Renamed Symbol |  |
| ND0111 | PE034A | * | Renamed Symbol |  |
| ND0112 | PE035A | * | Renamed Symbol |  |
| ND0113 | PE036A | * | Renamed Symbol |  |
| ND0114 | PE037A | * | Renamed Symbol |  |
| ND0115 | PE038A | * | Renamed Symbol |  |
| ND0116 | PE039A | * | Renamed Symbol |  |
| ND0117 | PE040A | * | Renamed Symbol |  |
| ND0118 | PE041A | * | Renamed Symbol |  |
| ND0119 | PE042A | * | Renamed Symbol |  |
| ND0120 | PE043A | * | Renamed Symbol |  |
| ND0121 | PF001A | * | Renamed Symbol |  |
| ND0122 | PF002A | * | Renamed Symbol |  |
| ND0123 | PF003A | * | Renamed Symbol |  |
| ND0125 | PF005A | * | Renamed Symbol |  |
| ND0126 | PF006A | * | Renamed Symbol |  |
| ND0127 | PF007A | * | Renamed Symbol |  |
| ND0124 | PF004A | * | Renamed Symbol |  |
| ND0128 | PF008A | * | Renamed Symbol |  |
| ND0129 | PF009A | * | Renamed Symbol |  |
| ND0131 | PF011A | * | Renamed Symbol |  |
| ND0132 | PF012A | * | Renamed Symbol |  |
| ND0133 | PF013A | * | Renamed Symbol |  |
| ND0130 | PF010A | * | Renamed Symbol |  |
| ND0134 | PF014A | * | Renamed Symbol |  |
| ND0135 | PF015A | * | Renamed Symbol |  |
| ND0136 | PP001A | * | Renamed Symbol |  |
| ND0137 | PP002A | * | Renamed Symbol |  |
| ND0138 | PP003A | * | Renamed Symbol |  |
| ND0139 | PP004A | * | Renamed Symbol |  |
| ND0140 | PP005A | * | Renamed Symbol |  |
| ND0141 | PP006A | * | Renamed Symbol |  |
| ND0142 | PP007A | * | Renamed Symbol |  |
| ND0143 | PP008A | * | Renamed Symbol |  |
| ND0144 | PP009A | * | Renamed Symbol |  |
| ND0145 | PP010A | * | Renamed Symbol |  |
| ND0146 | PP011A | * | Renamed Symbol |  |
| ND0147 | PP012A | * | Renamed Symbol |  |
| ND0148 | PP013A | * | Renamed Symbol |  |
| ND0149 | PP014A | * | Renamed Symbol |  |
| ND0150 | PP015A | * | Renamed Symbol |  |
| ND0151 | PP016A | * | Renamed Symbol |  |
| ND0152 | PP017A | * | Renamed Symbol |  |
| ND0156 | PS005A | * | Renamed Symbol |  |
| ND0157 | PS006A | * | Renamed Symbol |  |
| ND0158 | PS007A | * | Renamed Symbol |  |
| ND0153 | PS001A | * | Renamed Symbol |  |
| ND0154 | PS003A | * | Renamed Symbol |  |
| ND0155 | PS004A | * | Renamed Symbol |  |
| ND0159 | PS008A | * | Renamed Symbol |  |
| ND0160 | PS009A | * | Renamed Symbol |  |
| ND0161 | PS010A | * | Renamed Symbol |  |
| ND0162 | PS011A | * | Renamed Symbol |  |
| ND0163 | PS012A | * | Renamed Symbol |  |
| ND0164 | PS013A | * | Renamed Symbol |  |
| ND0165 | PS014A | * | Renamed Symbol |  |
| ND0166 | PS015A | * | Renamed Symbol |  |
| ND0167 | PS016A | * | Renamed Symbol |  |
| ND0168 | PT002A | * | Renamed Symbol |  |
| ND0169 | PT003A | * | Renamed Symbol |  |
| ND0170 | PT004A | * | Renamed Symbol |  |
| ND0171 | PT005A | * | Renamed Symbol |  |
| ND0172 | PT006A | * | Renamed Symbol |  |
| ND0173 | PT007A | * | Renamed Symbol |  |
| ND0174 | PT008A | * | Renamed Symbol |  |
| ND0175 | PT009A | * | Renamed Symbol |  |
| ND0176 | PT010A | * | Renamed Symbol |  |
| ND0177 | PT011A | * | Renamed Symbol |  |
| ND0178 | PV001A | * | Renamed Symbol |  |
| ND0179 | PV002A | * | Renamed Symbol |  |
| ND0180A | PV003A | * | Renamed Symbol |  |
| ND0180B | PV003B | * | Renamed Symbol |  |
| ND0181A | PV004A | * | Renamed Symbol |  |
| ND0181B | PV004B | * | Renamed Symbol |  |
| ND0182A | PV005A | * | Renamed Symbol |  |
| ND0182B | PV005B | * | Renamed Symbol |  |
| ND0183A | PV007A | * | Renamed Symbol |  |
| ND0183B | PV007B | * | Renamed Symbol |  |
| ND0184A | PV008A | * | Renamed Symbol |  |
| ND0184B | PV008B | * | Renamed Symbol |  |
| ND0185 | PV009A | * | Renamed Symbol |  |
| ND0186 | PV010A | * | Renamed Symbol |  |
| ND0187 | PV012A | * | Renamed Symbol |  |
| ND0188 | PV013A | * | Renamed Symbol |  |
| ND0189A | PV014A | * | Renamed Symbol |  |
| ND0189B | PV014B | * | Renamed Symbol |  |
| ND0190A | PV015A | * | Renamed Symbol |  |
| ND0190B | PV015B | * | Renamed Symbol |  |
| ND0191A | PV016A | * | Renamed Symbol |  |
| ND0191B | PV016B | * | Renamed Symbol |  |
| ND0192A | PV018A | * | Renamed Symbol |  |
| ND0192B | PV018B | * | Renamed Symbol |  |
| ND0193A | PV019A | * | Renamed Symbol |  |
| ND0193B | PV019B | * | Renamed Symbol |  |
| ND0196A | PV023A | * | Renamed Symbol |  |
| ND0196B | PV023B | * | Renamed Symbol |  |
| ND0197 | PZ002A | * | Renamed Symbol |  |
| ND0194 | PV021A | * | Renamed Symbol |  |
| ND0195 | PV022A | * | Renamed Symbol |  |
| ND0198 | PZ003A | * | Renamed Symbol |  |
| ND0199 | PZ004A | * | Renamed Symbol |  |
| ND0200 | PZ005A | * | Renamed Symbol |  |
| ND0201 | PZ006A | * | Renamed Symbol |  |
| ND0202 | PZ007A | * | Renamed Symbol |  |
| ND0203 | PZ008A | * | Renamed Symbol |  |
| ND0204 | PZ009A | * | Renamed Symbol |  |
| ND0205 | PZ010A | * | Renamed Symbol |  |
| ND0206A | PZ011A | * | Renamed Symbol |  |
| ND0206B | PZ011B | * | Renamed Symbol |  |
| ND0208 | PZ012A | * | Renamed Symbol |  |
| ND0209 | PZ013A | * | Renamed Symbol |  |
| ND0210 | PZ014A | * | Renamed Symbol |  |
| ND0211 | PZ015A | * | Renamed Symbol |  |
| ND0212 | PZ016A | * | Renamed Symbol |  |
| ND0213 | PZ017A | * | Renamed Symbol |  |
| ND0214 | PZ018A | * | Renamed Symbol |  |
| ND0215 | PZ019A | * | Renamed Symbol |  |
| ND0216 | PZ020A | * | Renamed Symbol |  |
| ND0217 | PZ021A | * | Renamed Symbol |  |
| ND0218 | PZ022A | * | Renamed Symbol |  |
| ND0219 | PZ023A | * | Renamed Symbol |  |
| ND0220 | PZ024A | * | Renamed Symbol |  |
| ND0221 | PZ025A | * | Renamed Symbol |  |
| ND0222 | PZ026A | * | Renamed Symbol |  |
| ND0223 | PZ027A | * | Renamed Symbol |  |
| ND0224 | STPL008 | * | Renamed Symbol |  |
| ND0248A | IM005A | * | Renamed Symbol |  |
| ND0248B | IM005B | * | Renamed Symbol |  |
| ND0249B | IM017B | * | Renamed Symbol |  |
| ND0249A | IM017A | * | Renamed Symbol |  |
| ND0249C | IM017C | * | Renamed Symbol |  |
| ND0225A | ND0225A | * | New Symbol |  |
| ND0225B | ND0225B | * | New Symbol |  |
| ND0226 | ND0226 | * | New Symbol |  |
| ND0227 | ND0227 | * | New Symbol |  |
| ND0228 | ND0228 | * | New Symbol |  |
| ND0236 | ND0236 | * | New Symbol |  |
| ND0237 | ND0237 | * | New Symbol |  |
| ND0238 | ND0238 | * | New Symbol |  |
| ND0239 | ND0239 | * | New Symbol |  |
| ND0240 | ND0240 | * | New Symbol |  |
| ND0241 | ND0241 | * | New Symbol |  |
| ND0242 | ND0242 | * | New Symbol |  |
| ND0243 | ND0243 | * | New Symbol |  |
| ND0244A | ND0244A | * | New Symbol |  |
| ND0244B | ND0244B | * | New Symbol |  |
| ND0245A | ND0245A | * | New Symbol |  |
| ND0245B | ND0245B | * | New Symbol |  |
| ND0246 | ND0246 | * | New Symbol |  |
| ND0247 | ND0247 | * | New Symbol |  |
| ND0250 | ND0250 | * | New Symbol |  |
| ND0251 | ND0251 | * | New Symbol |  |
| ND0252 | ND0252 | * | New Symbol |  |
| ND0253 | ND0253 | * | New Symbol |  |
| ND0254 | ND0254 | * | New Symbol |  |
| ND0255 | ND0255 | * | New Symbol |  |
| ND0256 | ND0256 | * | New Symbol |  |
| ND0259 | ND0259 | * | New Symbol |  |
| ND0260 | ND0260 | * | New Symbol |  |
| ND0261A | ND0261A | * | New Symbol |  |
| ND0261B | ND0261B | * | New Symbol |  |
| ND0262 | ND0262 | * | New Symbol |  |
| ND0263 | ND0263 | * | New Symbol |  |